As digits are received by senders, switching equipment move: into action by causing brass rods with multiple brushes or wipers to move upward on banks of term'-.pals to locate air available trunk to the office or number called. Other sender. operations then select the telephone number which was dialed and a connection through the office is completed. Ringing current is then applied to the called subscribers line and the ringing starts at the called subscriber's telephone set.

After establishing the connection to the called subscribers, the senders and associated equipment are released to handle .other calls.

Step by Step System

Step by Step is the oldest and simplest dialing sys :em.. It is a form of direct-dial control, in that switches are directly responsive to dialed digits for each stage of the dialing process.

The heart of this system is a relay operated switch with a vertical rod with horizontal wipers affixed to it. As a digit is dialed, the rod with wipers mowes up the banks of terminals the number of pulses of the digit dialed- The rod and wipers then rotates on the banks of terminals to an idle contact and connection to the next switch is made. Each succeeding digit dialed activities another switch, building a step-by-step connection through the central office. When all digits have been dialed, the connection is established and ringing current is applied to the line..

2436D3/bc/0006.0.0 	122

1